BIM Specification STRUC_NordicLCA_Housing_Concrete_BuildingPermit.ifc

| DISCIPLINE | Structural |
| --- | --- |
| Illustration image |  |
| Object |  |
| Intended use |  |
| Design phase | building permit |
| Revision date |  |
| Organisation | Gravicon Oy |
| BIM contact person |  |
| Contact email address |  |
| Contact telephone number |  |
| Principal designer |  |
| Project manager | Rita Lavikka |
| Software and versions used | Tekla Structures 2023 |
| Separate models and documents related to this model | STRUC_NordicLCA_Office_Concrete_BuildingPermit.ifc and Tekla Structures native model |
| Additional information and comments | It should be noted that this model has been developed primarily according to the principles of the BIM4LCA project and have been used to verify the principles of this project. This model is not flawless. Plans have not been thoroughly examined from a technical point of view, nor, for example, have the structural solutions been calculated or otherwise structurally dimensioned in detail. All structures are given as an example. Every IfcElementAssembly has a type with question mark (?) on Pset_ElementAssemblyCommon / Property:Reference. The question mark is replaced at production phase with a specific reference number. |

| GENERAL MODELLING PRINCIPLES |  |
| --- | --- |
| Classification and naming of building elements | Class; Name; Prefix. 1; DELTA_BEAM; B. 201; RECTANGLE_COLUMN; P. 204; RECTANGLE_BEAM; K. 210; HOLLOW_CORE_SLAB; O. 211; FLOOR_PLANK; KL. 214; LANDING; L. 214; SLAB; L. 221; INNER_SHELL; S. 222; SOCLE_PANEL; AS. 227; INNER_SHELL; SK. 228; INTERNAL_WALL; V. 250; BALCONY_SLAB; CL. 264; STAIR_ELEMENT; T. 301; FOOTING_CONTINUOUS; PV-JA. 302; FOOTING; PV-A. 305; FOUNDATION_WALL; PV-PM. 323; SOLID_SLAB; PV-L. 325; GROUND_SLAB; PV-ML. 326; SURFACE_SLAB; PV-PL. 342; INSULATION; ER. 349; FILL-IN_CONCRETE; PV-KAISTA. 350; RETAINING_WALL; PV-MP. 352; ROOF; VK. 352; TIMBER_TRUSS; KR. |
| Layer systems used | - |
| Unit system | Metric |
| Coordinate system | ETRS-GK25 planar coordinates N2000 elevation system |
| Origin location (x,y,z) | X = E = I Y = N = P Z Origin 25505171,787 6707188,716 0,000 Models are not rotated in the project coordinate system; all models are placed in a true-north position. |
| Level names and elevations in the N2000 system | Sea level +0,00 Foundations +47,300 Basement +49,070 Level_01 +52,000 Level_02 +54,930 Level_03 +58,030 Level_04 +61,130 Roof +64,035 |
| IFC version | IFC4 |
| Level of detail (LoD) | building permit |
| Deviations from the LoD |  |
| Level of information (LoI) | building permit |
| Deviations from the LoI |  |
| Presentation of the phasing | - |

|  | Propertyset | Property | Used values |
| --- | --- | --- | --- |
| Identification of the external envelope of the building | Pset________Common (part) | IsExternal | - Elements that are part of ou outside the outer envelope TRUE - Elements inside the outer envelope FALSE |
| Identification of load-bearing building elements | Pset________Common | IsLoadBearing | - Load-bearing elements TRUE - Non-load-bearing elements FALSE or empty |